# Computer Architecture Interview Guide

# 1. What is Computer Architecture?

Computer Architecture is how a computer is designed internally so it can execute programs efficiently.

It includes:

CPU

Memory

Cache

Buses

Registers

Input/Output devices

Think of it as the blueprint of a computer.

# 2. Basic CPU Execution

Suppose we write:

int c = a + b;

The CPU doesn't understand C++. The compiler converts it into machine instructions.

Then the CPU performs:

Read a ↓
Read b ↓
Add ↓
Store result

Every program eventually becomes these tiny operations.

# 3. CPU Pipeline ⭐⭐⭐⭐⭐

One of the most common interview topics.

Instead of doing one instruction at a time, the CPU overlaps multiple instructions.

Imagine a car factory. Instead of building one complete car...

Station 1: Install Engine → Station 2: Install Doors → Station 3: Paint → Station 4: Testing

Many cars move simultaneously. CPU does the same.

Pipeline stages:

Instruction → Fetch → Decode → Execute → Memory → Write Back

## Pipeline Stages Details

Fetch: CPU reads instruction from memory. (Example: ADD R1, R2)

Decode: CPU understands what instruction means. (Example: "Oh... This is ADD.")

Execute: ALU performs calculation. (Example: 5 + 10 = 15)

Memory: If instruction needs RAM, CPU reads or writes memory.

Write Back: Result stored into register. Done.

### Interview Question

Why do CPUs use pipelining?

Answer:

It improves performance by executing multiple instructions simultaneously at different stages rather than waiting for one instruction to complete before starting the next.

# 4. Pipeline Hazards (Surface Level)

Sometimes the pipeline gets stuck. There are three types of hazards:

## Data Hazard

Instruction 2 needs result from Instruction 1.

Example:
ADD R1, R2
SUB R3, R1

Second instruction cannot start until first finishes.

Solution: Forwarding or pipeline stall.

## Control Hazard

Occurs after if(), for(), while(), or a branch. CPU doesn't know which instruction comes next.

Solution: Branch Prediction.

## Structural Hazard

Two instructions need same hardware. Example: Two instructions want memory at same time. One must wait.

# 5. Registers ⭐⭐⭐⭐⭐

Registers are tiny memories inside CPU. They are very fast.

RAM → Register → ALU

CPU first copies data from RAM into registers, then performs calculations.

Registers are much faster than RAM.

### Interview Question

Why not store everything in registers?

Answer:

Registers are extremely fast but very small and expensive.

# 6. Memory Hierarchy ⭐⭐⭐⭐⭐

Speed decreases as we go down. Size increases as we go down.

Registers 
↓ 
L1 Cache 
↓ 
L2 Cache 
↓ 
L3 Cache 
↓ 
RAM 
↓ 
SSD/HDD

Higher = Faster | Lower = Bigger

# 7. Cache Memory ⭐⭐⭐⭐⭐

CPU is extremely fast, but RAM is much slower. So CPU keeps recently used data in cache.

Example:

Suppose program repeatedly accesses "temperature". Instead of fetching from RAM every time... CPU stores it in cache. Next access is much faster.

Three cache levels:

L1: Fastest, smallest, inside each core.

L2: Larger, slightly slower.

L3: Shared by all CPU cores. Largest cache.

### Interview Question

Why is cache needed?

Answer:

Cache stores frequently accessed data close to the CPU, reducing memory access time and improving performance.

Cache Terminology:

Cache Hit: Requested data found in cache. Very fast.

Cache Miss: Data not in cache. CPU goes to RAM. Slower.

# 8. RAM vs ROM

| RAM | ROM |
| --- | --- |
| Temporary Read + Write Loses data after power off Examples: Variables, Program execution | Permanent Stores firmware Doesn't lose data Examples: BIOS, Microcontroller firmware |

# 9. SRAM vs DRAM

Very common interview topic.

| SRAM (Static RAM) | DRAM (Dynamic RAM) |
| --- | --- |
| Fast Expensive Uses flip-flops No refresh needed Used for Cache | Cheaper Slower Needs refresh Used as Main Memory (RAM) |

### Interview Answer

Cache uses SRAM because it's faster. RAM uses DRAM because it's cheaper and can store much more data.

# 10. Stack vs Heap ⭐⭐⭐⭐

| Stack | Heap |
| --- | --- |
| Function variables (e.g., int x; int y;) Automatic memory management Small Very fast | Dynamic memory (e.g., new, malloc()) Programmer manages it (manual) Large Slower |

# 11. Virtual Memory

Programs think they have huge memory. Actually...

Program → Virtual Address → MMU → Physical RAM

OS converts virtual address into physical address.

Benefit: Programs don't interfere with each other (isolation/security).

# 12. Multi-Core Processor

Old CPU: 1 Core

Modern CPU: Core 1, Core 2, Core 3, Core 4

Multiple tasks run simultaneously.

Example:
Core 1: Browser
Core 2: Music
Core 3: Video
Core 4: Background apps

# 13. Bus ⭐⭐⭐⭐

A bus is simply a communication pathway. There are three important buses:

Data Bus: Transfers actual data (e.g., 10101010).

Address Bus: Tells memory "Where should I read/write?" (Example: Address 2000).

Control Bus: Carries control signals (Examples: Read, Write, Interrupt, Clock).

# 14. Microarchitecture (Just One Line)

Architecture defines what a processor should do (instruction set and behavior).

Microarchitecture defines how it is actually implemented internally (pipeline, caches, execution units, branch predictor, etc.).

Example: Intel and AMD processors can both support the x86 instruction set (same architecture) but have different microarchitectures.

# Questions They May Ask

Q: What is cache?
A: "A small, high-speed memory between the CPU and RAM that stores frequently accessed data."

Q: Why is cache faster than RAM?
A: "It is built using SRAM and is physically much closer to the CPU."

Q: Difference between SRAM and DRAM?
A: 
- SRAM: Faster, expensive, used for cache.
- DRAM: Slower, cheaper, used for main memory.

Q: What is pipelining?
A: "Executing multiple instructions simultaneously at different stages to improve CPU throughput."

Q: Why are registers needed?
A: "They store data that the CPU is currently processing because accessing registers is much faster than accessing RAM."

Q: What is a bus?
A: "A communication channel that transfers data, addresses, and control signals between computer components."