# OS, Embedded Basics, and Interview Tips

# 1. Operating Systems Basics ⭐⭐⭐⭐

Since she knows FreeRTOS and pthreads, interviewers may ask OS fundamentals.

## Process vs Thread

| Process | Thread |
| --- | --- |
| Independent program Own memory Heavyweight | Smallest execution unit Shares memory Lightweight |

## Context Switching

CPU switches from one task/thread to another.

Task A ↓
Task B ↓
Task C

## Synchronization

Why do we need Mutex or Semaphore?

To avoid Race Conditions and Deadlocks.

She already knows this because of FreeRTOS.

# 2. ARM Cortex Basics ⭐⭐⭐

Her resume mentions Cortex-M3.

ARM = RISC architecture
Low power
Widely used in embedded systems

## Difference:

ARM Cortex-M → Microcontrollers (ESP32, STM32)

ARM Cortex-A → Application processors (Phones, Linux systems)

# 3. RISC vs CISC ⭐⭐⭐⭐

| RISC | CISC |
| --- | --- |
| Simple instructions Fixed instruction size Faster execution Example: ARM | Complex instructions Variable instruction size Example: Intel x86 |

### Interview Answer

"ARM processors use a RISC architecture with simpler instructions for better power efficiency, whereas x86 processors use CISC with more complex instructions."

# 4. Memory-Mapped I/O ⭐⭐⭐

Very common embedded question. CPU communicates with peripherals using memory addresses.

Example:
0x40021000 → GPIO Register

Writing "GPIO = HIGH" actually writes to a memory address.

# 5. Interrupts ⭐⭐⭐⭐

Instead of continuously checking sensors, hardware interrupts CPU.

Example:
Button pressed → Interrupt → CPU immediately executes ISR.

Know the difference between Polling vs Interrupt.

# 6. DMA (Surface Level)

DMA = Direct Memory Access

Normally: Sensor → CPU → RAM

DMA: Sensor → RAM (CPU doesn't participate.)

Benefits: Faster, Lower CPU usage.

# 7. Hardware Debugging

Know tools:

Serial Monitor

Logic Analyzer

Oscilloscope

Even if she hasn't used an oscilloscope extensively, know what it is.

# 8. Git Basics

Since resume mentions Git. Know:

clone | commit | push | pull | branch | merge

# 9. Linux Basics ⭐⭐⭐⭐

## Commands

ls | cd | pwd | grep | cat | chmod | ps | top | kill

## Know Compilation Tools

gcc | g++ | make

Since Industrial project uses Makefile.

# 10. Basic OOP Revision

Even though C++ is done, revise:

Abstraction
Encapsulation
Inheritance
Polymorphism

Interviewers love: "What is virtual function?"

# 11. Communication Skills ⭐⭐⭐⭐⭐

She should answer using structure.

| Instead of: | Say: |
| --- | --- |
| "I used FreeRTOS." | "I built a four-task FreeRTOS application where different tasks handled sensing, display, and networking. Queues were used for inter-task communication and mutexes protected shared serial output." |

# Questions From Resume

These are more likely than SystemC. Expect:

Explain your Healthcare project.

Why Queue? Why Mutex?

Why Dual Core? Why ESP32?

How does MAX30102 work?

Explain pthread project.

Explain race condition.

Explain one bug you solved.

# Behavioral Questions

Prepare: Tell me about yourself. (2 minutes)
Prepare: Tell me about your internship.

## Why Infineon?

Good answer:

"Infineon is a leader in automotive, power electronics, and embedded semiconductor solutions. I'm interested in low-level software and embedded systems, and this role gives me an opportunity to work on SystemC-based virtual prototyping and next-generation SoC development while learning from experienced engineers."

## Why should we hire you?

Good answer:

"I have a strong embedded systems foundation through projects on ESP32, ARM Cortex-M3, Linux, and FreeRTOS. While technologies like SystemC are new to me, I'm comfortable learning new tools quickly, and I believe my problem-solving skills and firmware experience will help me contribute effectively."

## Tell me a difficult bug.

She has several:

Race condition in FreeRTOS
Shared serial buffer
Queue synchronization

Those make excellent stories.

# If They Ask Something Unknown

The best answer:

"I haven't worked with that technology directly, but based on my understanding..." followed by what she knows, and then "...I'd be excited to learn it in depth."

That demonstrates honesty and confidence.